Attached please find an installer for the LSXE/CQXE OCX.  These Active X files are required for the Hunter Color Quest XE and some other instruments.

Before you attempt to use the OCX please make sure that the instrument and computer are communicating by using a program such as Windows HyperTerminal.

Configure HyperTerminal as follows:

bits per second: 9600

data bits: 8

parity: none

stop bits: 1

flow control: none

Once connected, press the space bar on the PC. The sensor will reply with a string containing the version of its firmware.

Once you have confirmed communications, install the OCX and then run the attached VBScript file.

To install the OCX, copy the files on the CD to your Windows\System directory.  Locate the file called “LSXEOCX.EXE” and double click on it to install it.  This will install the Active X drivers and register them in the Windows registry so that the CyberChrome software knows where to find them.

The next step is to run the “COMMTEST.VBS” program to test the communications with the instrument.  Double click on this file to run it and communications with the instrument should be confirmed.

After this you are ready to communicate in the CyberChrome Color Systems software.